# Gravity, geometry and enclosure control the gas‑exchange boundary layer of *Arabidopsis*: a validated in‑browser CFD model linking spaceflight hardware to carbon gain

Richard J. Barker¹* and collaborators (author list provisional)

¹ MadWest / AstroBotany International Research Initiative (AIRI). Affiliations to be finalised. * Correspondence: admin@cosecloud.com

Manuscript draft for an npj Microgravity‑style methods + case‑study article. This document is

generated from the LunarLeaf‑CFD results package; figures (F1–F10) and tables (T1–T12) are in

`results/figures/` and `results/tables/`. A LaTeX version is in `results/manuscript/`.

# Abstract

Plants grown in spaceflight display stress phenotypes whose mechanistic basis remains debated. A long‑ suspected contributor is the loss of buoyancy‑driven convection in microgravity, which should thicken the unstirred boundary layer around plant organs and steepen the O₂/CO₂/H₂O gradients that govern gas exchange. We test this quantitatively with a browser‑based lattice‑Boltzmann model that solves incompressible flow, multi‑species advection–diffusion and Boussinesq buoyancy under an adjustable gravity vector. The solver is validated against four numerical benchmarks and anchored to measured Arabidopsis thaliana whole‑plant‑ chamber gas exchange (net assimilation 3.85 µmol CO₂ m⁻² s⁻¹). Reducing gravity from 1 g to microgravity removes convective ventilation (peak flow speed ∝ √g → 0) and steepens the surface gas gaps 1.7–2.3× at every scale; denser architectures — a rosette, then a microgreen canopy — trap air and amplify the effect. The forced airflow needed to restore Earth‑equivalent boundary layers rises with planting density (≈ 2.0 cm s⁻¹ for a leaf, ≈ 6.5 cm s⁻¹ for a rosette); a microgreen canopy is not restored within the solver's valid range. Mapping the three ISS growth systems onto dish boundary conditions, a sealed BRIC dish both drifts toward hypoxia and, once photosynthesis is made CO₂‑ limited, fixes only ≈ 1 % of the Earth carbon over a 12 h photoperiod; a micropore‑taped CARA dish sustains ≈ 90 % and a ventilated VEGGIE system ≈ 100 %. The model, runnable entirely in a web browser, provides a mechanistic bridge from cabin‑scale transport to plant carbon gain and a quantitative design tool for space‑flight growth hardware.

Keywords: microgravity, boundary layer, gas exchange, Arabidopsis, computational fluid dynamics, lattice‑Boltzmann, spaceflight hardware, photosynthesis.

# Introduction

On Earth a photosynthesising leaf warms and humidifies the thin layer of air in contact with its surface. The warmed, moistened air is less dense than the bulk atmosphere, becomes buoyant, and rises — a free‑ convection plume that continuously sweeps the leaf boundary layer, replenishing CO₂ and carrying away O₂ and water vapour. This buoyancy‑driven ventilation is invisible on Earth but does real physiological work: it sets the boundary‑layer conductance that, in series with stomatal and mesophyll conductances, limits gas exchange.

In microgravity there is no buoyancy. Density differences no longer generate motion, so in still air the only transport left is molecular diffusion. The unstirred boundary layer around a plant organ is expected to thicken, and the surface‑to‑bulk gradients of CO₂, O₂ and water vapour to steepen. This has been proposed for decades as a mechanism contributing to the altered morphology, gas exchange and stress signatures of spaceflight‑grown plants [1,2,11], and it is the stated rationale for the forced‑ventilation design of modern plant‑growth hardware. Yet the effect is difficult to measure directly: no whole‑chamber sensor resolves the sub‑millimetre boundary layer, and flight opportunities are scarce. A physically grounded, quantitative model is therefore valuable both to interpret existing results and to design hardware.

Here we build such a model and make it broadly usable — it runs entirely in a web browser with no installation. We (i) implement and validate an incompressible flow + multi‑species transport solver with a user‑controlled gravity vector; (ii) anchor it to measured Arabidopsis gas‑exchange rates; (iii) quantify how reduced gravity reshapes the boundary layer and its gas gradients across three biological scales — a single leaf, a rosette, and a dense microgreen canopy; (iv) compute the forced airflow required to counteract the microgravity penalty at each scale; (v) map the three most‑used ISS plant‑growth systems (BRIC, CARA, VEGGIE) onto solver boundary conditions; and (vi) close the loop by making photosynthesis CO₂‑ limited, translating the transport limitation into a prediction of carbon gain. Throughout we separate what the data validate (the flux magnitude and closed‑chamber transport) from what the model predicts (the spatial, gravity‑ and scale‑dependent gradients).

# Results

### Solver validation

Before any biology, the incompressible‑flow + advection–diffusion solver was verified against four independent benchmarks (Table 1). It reproduces the Ghia et al. [3] lid‑driven‑cavity centreline profiles at Re 100 (L2 = 0.011), the Kármán vortex‑shedding Strouhal number of a confined cylinder (St = 0.192), the de Vahl Davis [4] differentially‑heated‑cavity Nusselt number at Ra 10⁴ (Nu = 2.242 vs 2.238, 0.18 % error), and the analytic error‑function profile for pure diffusion (L2 ≈ 0). Gate 3 is decisive: reproducing the natural‑convection benchmark to 0.18 % confirms that the buoyancy/gravity coupling is quantitatively correct, so scaling the gravity vector to zero genuinely collapses convection to the diffusion limit certified by gate 4.

Table 1 | Numerical validation gates.

| Gate | Benchmark | Reference | Model | Result |
| --- | --- | --- | --- | --- |
| 1 | Lid‑driven cavity, Re 100 | Ghia et al. (1982) [3] | L2 = 0.011 | ✓ |
| 2 | Cylinder wake, Re 100 | St ≈ 0.16 (unconfined) | St = 0.192 (confined) | ✓ |
| 3 | Natural‑convection cavity, Ra 10⁴ | de Vahl Davis (1983) [4] Nu = 2.238 | Nu = 2.242 | ✓ |
| 4 | Pure diffusion | ½·erfc analytic | L2 ≈ 0 | ✓ |

### Measured gas exchange anchors the model

Two independent measurements fix the physical stomatal flux (Fig. 1; Table 2). A whole‑plant‑chamber dataset [5] gives a net CO₂ assimilation of 1.38 µmol CO₂ h⁻¹ cm⁻² = 3.85 µmol m⁻² s⁻¹ for Col wild‑type, with dark respiration 1.18 µmol m⁻² s⁻¹ (R/A = 0.31) — textbook‑typical for Arabidopsis. An independent 4.8‑day Vernier whole‑chamber trace (Fig. 1a) shows the expected closed‑system signature: strong diel O₂ cycling (6.6–32.8 %) and humidity swings driven by photosynthesis and respiration. These fix the model's surface source/sink terms in physical units.

Figure 1 | Measured whole‑chamber gas exchange. Vernier closed‑chamber trace over 4.8 days: O₂, temperature, relative humidity and absolute humidity, showing diel gas cycling driven by the plant.

Figure 2 | One diel O₂ cycle with peak photosynthesis and respiration slopes, from which whole‑chamber gas‑exchange fluxes are derived.

Table 2 | Measured gas exchange (selected).

| Quantity | Value | Units | Source |
| --- | --- | --- | --- |
| Net CO₂ assimilation A | 1.38 (3.85) | µmol h⁻¹ cm⁻² (µmol m⁻² s⁻¹) | Chamber [5] |
| Dark respiration R | 0.43 (1.18) | µmol h⁻¹ cm⁻² (µmol m⁻² s⁻¹) | Chamber [5] |
| Chamber volume / area | 339 / 26 | cm³ / cm² | Chamber [5] |
| Vernier O₂ diel range | 6.6–32.8 | % | Vernier |

### Calibration and closed‑chamber transport

Resolving the leaf at 52 lattice cells across a 1.5 cm blade maps the lattice to physical units as dx = 0.288 mm, dt = 0.173 ms, giving an Earth‑gravity convective velocity of ≈ 7.7 cm s⁻¹ near the leaf — a physically reasonable free‑convection speed (Table 3). Run as a sealed chamber with a central source, the solver reproduces monotonic gas accumulation and conserves mass to a relative error of 2 × 10⁻⁵ (Fig. 3a), matching the imposed flux exactly. A persistent surface‑to‑bulk concentration gap develops between the near‑ leaf air and the far wall (Fig. 3b): the boundary layer expressed at chamber scale, and the quantity that gravity modulates below.

Figure 3 | Closed‑chamber transport validation. (a) Sealed‑chamber total gas matches the imposed flux exactly (mass conserved to 2 × 10⁻⁵). (b) A sustained near‑leaf vs far‑wall concentration gap — the boundary layer at chamber scale.

### Gravity controls the boundary layer

Sweeping gravity from Earth through Mars (0.38 g) and the Moon (0.17 g) to microgravity, convective ventilation weakens monotonically (peak speed ∝ √g) while the surface gas gaps steepen (Fig. 4a). The field maps make the mechanism visible (Fig. 5): at 1 g a buoyant plume rises off the leaf, thinning the upper‑ surface boundary layer; in microgravity the plume is gone and a symmetric, thicker diffusive halo surrounds the blade. The surface CO₂ gap roughly doubles from Earth to microgravity for an isolated leaf.

Figure 4 | Gravity and scale. (a) Single‑leaf gravity sweep: surface gaps rise and convection falls as gravity decreases. (b) Scale amplification and the microgravity penalty across leaf, rosette and canopy.

Figure 5 | Water‑vapour boundary layer. Buoyant plume (leaf, Earth), symmetric stagnant halo (leaf, microgravity), and trapped within‑canopy air (microgreen canopy, microgravity).

### Scale amplifies the effect

Moving from a single leaf to a rosette to a microgreen stand compounds the gravity penalty (Fig. 4b). Over‑ lapping rosette leaves trap air in the crown, producing the steepest local gradients of any geometry; the dense canopy ventilates only at its top, so the within‑canopy air stagnates as a whole (highest mean gradient). Both effects intensify in microgravity, making a microgravity canopy the worst case.

### Physical prediction of surface gas drawdown

TODO (2026-10-07): Table 4 and the ppm values in this paragraph come from `T5`, which was not

regenerated after the steady-state rerun (it needs the raw Vernier data in `validation/raw/`). The

microgravity drawdowns will rise; re-run `validation/analyze_data.py` and `analyze_model.py`, then update.

Anchoring the model's dimensionless gradients on the measured assimilation flux and a standard still‑air leaf boundary‑layer conductance (1.0 mol m⁻² s⁻¹) yields concrete predictions (Table 4): a boundary‑layer CO₂ drawdown of ≈ 4 ppm for an isolated leaf on Earth, rising to ≈ 13 ppm for the rosette bulk in microgravity, with trapped crown pockets reaching ≈ 25 ppm. These boundary‑layer contributions add to the larger stomatal and mesophyll drops.

Table 4 | Predicted leaf‑surface CO₂ drawdown (ppm), anchored on measured flux.

| Scenario | Mean | Peak |
| --- | --- | --- |
| Leaf · Earth | 3.8 | 5.9 |
| Leaf · microgravity | 7.8 | 8.9 |
| Rosette · Earth | 7.1 | 17.1 |
| Rosette · microgravity | 13.2 | 24.7 |
| Canopy · Earth | 7.8 | 12.9 |
| Canopy · microgravity | 10.6 | 14.4 |

### Forced ventilation reverses the penalty — but the requirement scales with density

Because the microgravity penalty is transport‑limited, it is engineerable: forced airflow substitutes for the missing buoyant convection. For an isolated leaf, a fan of only ≈ 2.0 cm s⁻¹ restores Earth‑equivalent surface gradients (Fig. 6). Extending the sweep to all three scales exposes a result with direct hardware consequences (Fig. 7a): the Earth‑equivalent airflow rises with planting density — ≈ 2.0 cm s⁻¹ for a leaf and ≈ 6.5 cm s⁻¹ for a rosette (Fig. 7b), about 3×. The canopy is the hardest to ventilate: side airflow skims the top of the stand while the within‑canopy air stays comparatively stagnant, so its gradient‑vs‑speed curve levels off above the Earth level and does not reach it within the solver's valid range (Mach ≤ 0.3, up to 6.6 cm s⁻¹); faster runs exceed that limit or become numerically unstable. This is a hint that a real dense canopy needs vigorous, likely turbulent, through‑canopy flow, and resolving it needs a finer lattice.

Figure 6 | Forced ventilation nulls the microgravity penalty (single leaf). Surface gap vs fan speed, with the Earth‑1 g and µg‑no‑fan references and the ≈ 2.0 cm s⁻¹ Earth‑equivalent point.

Figure 7 | Ventilation requirement scales with planting density. (a) Fan‑sweep curves for leaf/rosette/ canopy with each scale's Earth reference and crossing point; hollow points exceed Mach 0.3 and are not used. (b) Earth‑equivalent airflow ≈ 2.0 / 6.5 cm s⁻¹ for leaf / rosette; the canopy does not reach its Earth level within the valid range (≤ 6.6 cm s⁻¹).

### Spaceflight hardware as boundary conditions

The three most‑used ISS plant systems differ, in gas‑transport terms, only in the boundary condition they impose on the Petri dish (Fig. 8; Fig. 9). BRIC — hermetically sealed [6] — imposes zero‑flux walls: the enclosure atmosphere drifts without bound (Fig. 8a), and a simple mass balance on the small dish reservoir (Table 5) shows the ~400 ppm of CO₂ in a 30 cm³ headspace is fixed by photosynthesis in ≈ 7 minutes (light), while in the dark CO₂ reaches a stressful 1 % in ≈ 9 hours and O₂ falls to a hypoxic 5 % in ≈ 6.5 days. BRIC also gives the steepest leaf‑surface gradient of any hardware (Fig. 8b). CARA — gas‑permeable micropore surgical tape⁸ — vents the dish to the effectively‑infinite cabin, so the dish‑mean stays a few ppm from ambient (consistent with ground measurements that surgical tape keeps plated seedlings near‑ambient while parafilm and plastic wrap deplete CO₂ and trigger thousands of stress genes [8,9]); but it does not fix the microgravity boundary layer, so the leaf‑surface gradient remains that of an open dish (Fig. 9). VEGGIE — light + forced airflow — vents the enclosure and thins the leaf boundary layer, cutting the surface gradient ~4× relative to an open microgravity dish. Across all three scales, BRIC ≈ CARA at the leaf surface. A single VEGGIE fan speed (~8 cm s⁻¹) over‑ventilates a leaf and brings a rosette to about its Earth level (|ΔC CO₂| 0.294 vs 0.298), but does not restore a canopy, which no fan speed within the solver's valid range brings back to its Earth level (Fig. 9).

Figure 8 | Spaceflight hardware as dish boundary conditions. (a) Enclosure‑mean CO₂ drift over the 6.9 s (40 k‑step) run — BRIC diverges fastest (light depletes, dark accumulates); the taped CARA dish drifts at about two‑thirds of BRIC's rate, and whether it levels off is not resolved in this window. (b) Leaf‑surface gradient by hardware.

Figure 9 | Hardware × plant scale (microgravity). BRIC ≈ CARA at the leaf surface at every scale; one VEGGIE fan speed restores leaf and rosette but not the canopy. BRIC/CARA are from the original 3.8 s run (BRIC has no steady state); VEGGIE is at steady state; the hatched canopy VEGGIE bar exceeds Mach 0.3.

Table 5 | Sealed‑dish (BRIC) atmosphere timescales (analytic, 30 cm³ dish, 3 cm² leaf).

| Event | Time |
| --- | --- |
| Light: CO₂ 400 → ~0 ppm (photosynthesis self‑limits) | ≈ 7 min |
| Dark: CO₂ → 1 % (stress) | ≈ 9 h |
| Dark: O₂ 21 → 5 % (hypoxia) | ≈ 6.5 d |

### Closing the loop: CO₂‑limited photosynthesis

The analyses above impose a fixed assimilation flux. Because photosynthesis is CO₂‑limited, the depletion the boundary layer creates should feed back and suppress the flux. Making the leaf's CO₂ uptake follow a compensation‑point + saturation response of the local surface CO₂, the boundary‑layer depletion self‑ limits assimilation by ≈ 1–4 % at steady state, tracking the transport story: smallest under ventilation (VEGGIE, net A = 99.3 % of potential), larger for a bare microgravity leaf (98.0 %), and largest in the trapped rosette crown (96.5 %) (Table 6). Coupling the same response to the enclosure mass balance and the microgravity boundary‑layer conductance over a 12 h photoperiod (Fig. 10) exposes the growth consequence: in a sealed BRIC dish the surface CO₂ falls to the compensation point within minutes, photosynthesis collapses, and only ≈ 1 % of the Earth carbon is fixed; the taped CARA dish sustains ≈ 90 % and the ventilated VEGGIE system ≈ 100 %. The transport limitations are therefore not merely gradients — they throttle carbon gain, mildly where air moves and almost completely where it is sealed and still.

Figure 10 | Closed‑loop CO₂‑limited photosynthesis. (a) Net assimilation over a photoperiod — the sealed BRIC dish collapses within minutes. (b) Integrated 12 h carbon fixed: BRIC ≈ 1 %, CARA ≈ 90 %, VEGGIE ≈ 100 % of Earth.

Table 6 | Closed‑loop net assimilation (solver, % of light‑saturated potential).

| Scenario | Net A (%) |
| --- | --- |
| Leaf · Earth | 98.9 |
| Leaf · microgravity | 98.0 |
| Rosette · microgravity (crown) | 96.5 |
| Canopy · microgravity | 97.5 |
| Leaf · µg · VEGGIE | 99.3 |

# Discussion

The results give a mechanistic, quantitative account of a long‑suspected spaceflight stress: without buoyancy‑driven convection, plant organs sit in a thickened, poorly‑ventilated boundary layer that is simultaneously CO₂‑starved (suppressing photosynthesis), O₂‑enriched (promoting photorespiration) and humid (reducing transpirational cooling and nutrient transport). Crucially, this is not a fixed penalty but one that scales with both gravity and canopy architecture, and — through the CO₂ feedback — with the gas‑exchange control of the enclosure. The denser the planting and the more sealed the container, the more a still, low‑ gravity atmosphere throttles carbon gain.

Because the effect is transport‑ rather than biochemistry‑limited, it is engineerable, and the same solver that quantifies the penalty sizes the forced airflow needed to remove it. The scale dependence of that requirement (≈ 2.0 → 6.5 cm s⁻¹ from leaf to rosette, with the canopy not restored within the solver's valid range) is a concrete design guideline: a ventilation setting or permeable seal tuned on sparse plants will under‑serve a dense stand. The hardware mapping frames the three ISS systems as a ladder of increasing gas‑exchange control — sealed (both enclosure drift and surface layer), vented seal (enclosure fixed, surface not), actively ventilated (both fixed) — and the photosynthesis feedback converts these into a growth currency: a sealed dish fixes almost no carbon over a photoperiod. This offers a parsimonious, physics‑based contributor to the reduced growth and stress transcriptomes reported for sealed‑hardware spaceflight experiments [6,9], complementary to gravisensing‑ and light‑driven responses [7]. Consistent with a transport-limited mechanism, whole-stand gas exchange in microgravity is unchanged at saturating CO₂ [10] — exactly the regime in which the boundary-layer CO₂ limitation is relieved. Notably, a meta-analysis of 15 Arabidopsis spaceflight transcriptome experiments found that flight hardware and lighting impose some of the largest confounding effects on the response, with hypoxia and oxidative-stress signatures among its conserved features [11]; the present model provides a physical basis for that hardware dependence, and its BRIC-specific O₂ drawdown (§3.7) is consistent with the hypoxia signatures reported for sealed-canister experiments.

Limitations. The model is 2‑D and uses the Boussinesq approximation; results are kept within its validity (β·ΔC ≲ 0.5), and Earth‑equivalent fan speeds use only runs below Mach 0.3 (u_max ≲ 0.17 lattice units). The canopy VEGGIE value (Mach 0.32) is just outside that range. Photosynthesis is CO₂‑limited through a rectangular‑ hyperbola response, but stomatal conductance is not yet dynamic (transpiration is held CO₂‑independent), and the CO₂‑response and tape‑permeance parameters are reasonable assumptions rather than fits. Absolute ΔC values are reported in lattice excess units and converted to ppm through a single literature‑anchored conductance; the microgravity boundary layer is further bounded by the finite chamber, so the reported gravity ratios are conservative lower bounds. Finally, the ground datasets validate the flux magnitude and closed‑chamber transport, not the spatial boundary layer directly — which no whole‑chamber sensor can resolve — so the gravity‑ and scale‑dependent gradients remain model predictions built on validated components.

Future work. (i) Make stomatal conductance dynamic (CO₂‑ and humidity‑responsive) so transpiration and assimilation co‑vary; (ii) enlarge the domain and move to 3‑D (WebGPU) to remove the chamber‑bound conservatism and let a dense canopy develop the turbulent through‑flow its ventilation requires; (iii) calibrate against spatially‑resolved surface‑gas imaging (e.g. planar‑optode pH/CO₂/O₂ maps) for a direct test of the predicted gradients; (iv) couple to a whole‑plant carbon‑balance model to translate per‑ photoperiod carbon deficits into growth‑rate predictions across mission durations.

# Methods

Flow solver. Incompressible flow is solved with a two‑dimensional D2Q9 Bhatnagar–Gross–Krook (BGK) lattice‑Boltzmann scheme with halfway bounce‑back walls. The relaxation time τ sets the kinematic viscosity via ν = c_s²(τ − ½) with c_s² = ⅓ in lattice units. Body forces (buoyancy) are applied with the Guo forcing scheme.

Species transport. Three gas species (CO₂, O₂, H₂O) are transported by D2Q5 advection–diffusion lattices that share the flow velocity, with molecular diffusivities in the ratio 1.6 : 2.0 : 2.4 (× 10⁻⁵ m² s⁻¹). Walls are zero‑flux (bounce‑back), fixed‑concentration (anti‑bounce‑back Dirichlet) or semi‑permeable membrane as required by the scenario.

Gravity and buoyancy. Gravity enters through a Boussinesq body force f = ρ g Σₖ βₖ(cₖ − cₖ,₀), where g is a user‑set gravity vector (0 → 9.81 m s⁻²) and βₖ are signed density‑change coefficients (water vapour and warm air lighter; CO₂ heavier). Setting g = 0 removes the force, collapsing transport to diffusion.

Boundary conditions and hardware. The leaf surface imposes a stomatal source/sink (CO₂ uptake, O₂ + H₂O release; signs reversed and scaled by R/A = 0.31 in the dark). Chamber walls are ambient‑held (Dirichlet), sealed (zero‑flux, BRIC), semi‑permeable (the lid/side cells relax toward ambient each step, CARA micropore tape), or an inlet/outlet ventilation pair (VEGGIE forced airflow). The dense canopy uses a reduced per‑leaf flux to represent self‑shading.

Photosynthesis feedback. When enabled, the CO₂ uptake follows A(C) = A_max (C − Γ)/(C − Γ + K) of the local surface CO₂ (Γ compensation point, K half‑saturation), normalised to the measured flux at ambient, so boundary‑layer depletion self‑limits assimilation; O₂ release tracks A.

Validation, calibration and the 0‑D enclosure model. The solver was validated on four benchmarks (Table 1). The lattice was mapped to physical units by resolving a 1.5 cm leaf at 52 cells and matching the H₂O diffusivity. A 0‑D coupled enclosure–photosynthesis model integrates the CO₂ response, the boundary‑ layer conductance (Earth free convection vs microgravity diffusion) and the enclosure exchange over a 12 h photoperiod.

Data sources. Measured gas exchange: Vernier whole‑chamber logs and a whole‑plant‑chamber biomass/gas‑ exchange workbook [5]. All model code, the validation harness and the analysis scripts are in the LunarLeaf‑CFD repository; runs are 30 000 (or 22 000) steps to quasi‑steady state.

# Data and code availability

Model source, validation harness, analysis scripts and this results package are in the LunarLeaf‑CFD repository. Measured gas‑exchange data comprise Vernier whole‑chamber logs and the whole‑plant‑chamber workbook of Chew, Y.H. & Millar, A.J. [5]; raw third‑party data are referenced by provenance and not redistributed here.

# Author contributions

R.J.B. conceived the study, built the model and analysis, and wrote the manuscript. Co‑author contributions to be added.

# Competing interests

The authors declare no competing interests.

# Acknowledgements

Measured gas‑exchange data courtesy of Y.H. Chew and A.J. Millar (University of Edinburgh). (To be completed.)

# References

Kitaya, Y., Tsuruyama, J., Shibuya, T., Yoshida, M. & Kiyota, M. Effects of air current speed on gas exchange in plant leaves and plant canopies. Adv. Space Res. 31, 177–182 (2003). doi:10.1016/S0273-1177(02)00747-0.

Porterfield, D. M. The biophysical limitations in physiological transport and exchange in plants grown in microgravity. J. Plant Growth Regul. 21, 177–190 (2002). doi:10.1007/s003440010054.

Ghia, U., Ghia, K. N. & Shin, C. T. High-Re solutions for incompressible flow using the Navier–Stokes equations and a multigrid method. J. Comput. Phys. 48, 387–411 (1982). doi:10.1016/0021-9991(82)90058-4.

de Vahl Davis, G. Natural convection of air in a square cavity: a bench mark numerical solution. Int. J. Numer. Methods Fluids 3, 249–264 (1983). doi:10.1002/fld.1650030305.

TODO (2026-10-07): unverified reference. As cited: Chew, Y. H., Seaton, D. D., Mengin, V., Flis, A., Mugford, S. T., Smith, A. M., Stitt, M. & Millar, A. J. Linking circadian time to growth rate quantitatively via carbon metabolism. bioRxiv 105437 (2017). doi:10.1101/105437. CrossRef resolves this DOI to a different title, "The Arabidopsis Framework Model version 2 predicts the organism-level effects of circadian clock gene mis-regulation", with 13 authors (Chew, Seaton, Mengin, Flis, Mugford, George, Moulin, Hume, Zeeman, Fitzpatrick, Smith, Stitt, Millar). Confirm which publication holds the gas-exchange dataset and correct the title, authors or DOI. (Whole-plant-chamber gas-exchange dataset used here.)

Correll, M. J., Pyle, T. P., Millar, K. D. L., Sun, Y., Yao, J., Edelmann, R. E. & Kiss, J. Z. Transcriptome analyses of Arabidopsis thaliana seedlings grown in space: implications for gravity-responsive genes (BRIC hardware). Planta 238, 519–533 (2013). doi:10.1007/s00425-013-1909-x.

Zhou, M., Ferl, R. J. & Paul, A.-L. Light has a principal role in the Arabidopsis transcriptomic response to the spaceflight environment (CARA). npj Microgravity 10, 82 (2024). doi:10.1038/s41526-024-00417-0.

Ma, Y., Li, F., Wang, X., Sun, Q., Wang, R. & Zhao, J. Beware of sealing film of Petri dishes!—alters the expression of a large number of genes. Int. J. Mol. Sci. 26, 5484 (2025). doi:10.3390/ijms26125484.

Xu, L., Li, S., Shabala, S., Jian, T. & Zhang, W. Plants grown in parafilm-wrapped Petri dishes are stressed and possess altered gene-expression profiles. Front. Plant Sci. 10, 637 (2019). doi:10.3389/fpls.2019.00637.

Monje, O., Stutte, G. W. & Chapman, D. K. Microgravity does not alter plant stand gas exchange of wheat at moderate light levels and saturating CO₂ concentration. Planta 222, 336–345 (2005). doi:10.1007/s00425-005-1529-1.

Barker, R., Kruse, C. P. S., Johnson, C., Saravia-Butler, A., Fogle, H., Chang, H.-S., Trane, R. M., Kinscherf, N., Villacampa, A., Manzano, A., Herranz, R., Davin, L. B., Lewis, N. G., Perera, I., Wolverton, C., Gupta, P., Jaiswal, P., Reinsch, S. S., Wyatt, S. & Gilroy, S. Meta-analysis of the space flight and microgravity response of the Arabidopsis plant transcriptome. npj Microgravity 9, 21 (2023). doi:10.1038/s41526-023-00247-6.